Copyright 2015 by The American Institute of Architects (AIA)

Exclusively published and distributed by AVITRU, LLC for the AIA

This Product MasterSpec Section is licensed by AVITRU, LLC to Oatey ("Licensee").

This Product MasterSpec Section modifies the original MasterSpec text and does not include the full content of the original MasterSpec Section.

Revisions made to the original MasterSpec text are made solely by the Licensee and are not endorsed by, or representative of the opinions of, Avitru or The American Institute of Architects (AIA). Neither AIA nor Avitru are liable in any way for such revisions or for the use of this Product MasterSpec Section by any end user. A qualified design professional should review and edit the document to suit project requirements.

For more information, contact Oatey, 20600 Emerald Pkwy, Cleveland, OH 44135; Phone: (216) 267-7100; Website: www.oatey.com; Email: info@oatey.com.

For information about MasterSpec, contact Avitru at (800) 424-5080 or visit www.avitru.com.

SECTION 224100 - RESIDENTIAL PLUMBING FIXTURES

TIPS:

To view non-printing Editor's Notes that provide guidance for editing, click on MasterWorks/Single-File Formatting/Toggle/Editor's Notes.

To read detailed research, technical information about products and materials, and coordination checklists, click on MasterWorks/Supporting Information.

Revise this Section by deleting and inserting text to meet Project-specific requirements.

Verify that Section titles referenced in this Section are correct for this Project's Specifications; Section titles may have changed.

- GENERAL

      - RELATED DOCUMENTS

Retain or delete this article in all Sections of Project Manual.

        - Drawings and general provisions of the Contract, including General and Supplementary Conditions and Division 01 Specification Sections, apply to this Section.

      - SUMMARY

        - Section Includes:

          - Bathtubs.

          - Faucets.

          - Whirlpool baths.

          - Walk-in baths.

          - Bidets.

          - Lavatories.

          - Showers.

          - Bar sinks.

          - Kitchen sinks.

          - Laundry trays.

          - Dishwasher air-gap fittings.

          - Laminar-flow, faucet-spout outlets.

          - Disposers.

          - Hot-water dispensers.

          - Water closets.

          - Toilet seats.

          - Supply fittings.

          - Waste fittings.

        - Related Requirements:

Retain subparagraphs below to cross-reference requirements Contractor might expect to find in this Section but are specified in other Sections.

          - Section 224213.13 "Commercial Water Closets."

          - Section 224213.16 "Commercial Urinals."

          - Section 224216.13 "Commercial Lavatories."

          - Section 224216.16 "Commercial Sinks."

          - Section 224223 "Commercial Showers."

          - Section 224300 "Medical Plumbing Fixtures."

          - Section 224500 "Emergency Plumbing Fixtures."

          - Section 224600 "Security Plumbing Fixtures."

          - Section 224713 "Drinking Fountains."

          - Section 224716 "Pressure Water Coolers."

          - Section 224723 "Remote Water Coolers."

      - ACTION SUBMITTALS

        - Product Data: For each type of product.

          - Include construction details, material descriptions, dimensions of individual components and profiles, and finishes for lavatories.

          - Include rated capacities, operating characteristics, electrical characteristics, and furnished specialties and accessories.

        - Sustainable Design Submittals:

"Product Data" Subparagraph below applies to LEED (including LEED v4), IgCC, and Green Globes for fixtures and equipment that consume water.

          - Product Data: For water consumption.

Retain "Shop Drawings" Paragraph below if equipment includes wiring.

        - Shop Drawings: Include diagrams for power, signal, and control wiring.

      - INFORMATIONAL SUBMITTALS

Retain "Coordination Drawings" Paragraph below where limited space necessitates maximum utilization for efficient installation of different components or if coordination is required for installation of products and materials by separate installers.

        - Coordination Drawings: Counter cutout templates for mounting of counter-mounted plumbing fixtures.

        - Sample Warranty: For special warranty.

      - CLOSEOUT SUBMITTALS

        - Operation and Maintenance Data: For plumbing fixtures and faucets to include in emergency, operation, and operation and maintenance manuals.

Retain subparagraph below if whirlpool baths or walk-in baths are required for Project.

          - In addition to items specified in Section 017823 "Operation and Maintenance Data," include the following:

            - Servicing and adjustments of [whirlpool baths] [and] [walk-in baths].

      - MAINTENANCE MATERIAL SUBMITTALS

        - Furnish extra materials that match products installed and that are packaged with protective covering for storage and identified with labels describing contents.

          - Faucet Washers and O-Rings: Equal to [10] <Insert number> percent of amount of each type and size installed.

          - Faucet Cartridges and O-Rings: Equal to [5] <Insert number> percent of amount of each type and size installed.

          - Flushometer-Tank Repair Kits: Equal to [5] <Insert number> percent of amount of each type installed, but no fewer than [two] <Insert number> of each type.

          - Toilet Seats: Equal to [5] <Insert number> percent of amount of each type installed.

      - WARRANTY

When warranties are required, verify with Owner's counsel that warranties stated in this article are not less than remedies available to Owner under prevailing local laws.

        - Special Warranty: Manufacturer agrees to repair or replace components of [walk-in baths] [and] [whirlpool baths] that fail in materials or workmanship within specified warranty period.

          - Failures include, but are not limited to, the following:

            - Structural failures of unit shell.

            - Faulty operation of controls, blowers, pumps, heaters, and timers.

            - Deterioration of metals, metal finishes, and other materials beyond normal use.

Verify available warranties and warranty periods for units and components.

          - Warranty Period for Residential Applications of Shells: [Five] [20] [30] <Insert number> years from date of Substantial Completion.

          - Warranty Period for Residential Applications of Pumps and Blowers: [Five] [20] <Insert number> years from date of Substantial Completion.

          - Warranty Period for Residential Applications of Electronic Controls: [Five] <Insert number> years from date of Substantial Completion.

- PRODUCTS

Manufacturers and products listed in SpecAgent and MasterWorks Paragraph Builder are neither recommended nor endorsed by the AIA or Avitru. Before inserting names, verify that manufacturers and products listed there comply with requirements retained or revised in descriptions and are both available and suitable for the intended applications. For definitions of terms and requirements for Contractor's product selection, see Section 016000 "Product Requirements."

      - BATHTUBS

Copy "Bathtubs" Paragraph below and re-edit for each product.

Insert drawing designation. Use these designations on Drawings to identify each bathtub.

        - Bathtubs <Insert drawing designation>: [Enameled cast iron] [Enameled steel] [FRP] [PMMA], with shower.

          - Enameled Cast-Iron Bathtubs:

Retain "Manufacturers" Subparagraph and list of manufacturers below to require products from manufacturers listed or a comparable product from other manufacturers.

            - Manufacturers: Subject to compliance with requirements, provide products by the following:

              - Kohler Co.

              - <Insert manufacturer's name>.

          - Enameled-Steel Bathtubs:

Retain "Manufacturers" Subparagraph and list of manufacturers below to require products from manufacturers listed or a comparable product from other manufacturers.

            - Manufacturers: Subject to compliance with requirements, provide products by one of the following:

              - American Standard.

              - Bootz Industries.

              - Briggs Plumbing Products, Inc.

              - Crane Plumbing, L.L.C.

              - Eljer, Inc.

              - <Insert manufacturer's name>.

          - FRP Bathtubs:

Retain "Manufacturers" Subparagraph and list of manufacturers below to require products from manufacturers listed or a comparable product from other manufacturers.

            - Manufacturers: Subject to compliance with requirements, provide products by one of the following:

              - American Standard.

              - Clarion Bathware.

              - Florestone Products Co., Inc.

              - Praxis Industries, LLC.

              - Sterling.

              - Swan Corporation (The).

              - <Insert manufacturer's name>.

          - PMMA (Acrylic) Bathtubs:

Retain "Manufacturers" Subparagraph and list of manufacturers below to require products from manufacturers listed or a comparable product from other manufacturers.

            - Manufacturers: Subject to compliance with requirements, provide products by one of the following:

              - Acryline USA, Inc.

              - American Standard.

              - Aqua Bath Company, Inc.

              - Aqua Glass Corporation.

              - Aquatic Industries, Inc.

              - Clarion Bathware.

              - Crane Plumbing, L.L.C.

              - Florestone Products Co., Inc.

              - Kohler Co.

              - Lyons Industries, Inc.

              - Mansfield Plumbing Products LLC.

              - Praxis Industries, LLC.

              - St. Thomas Creations.

              - Swan Corporation (The).

              - <Insert manufacturer's name>.

          - Fixture:

Retain one of two "Standard" subparagraphs below.

            - Standard: ASME A112.19.1/CSA B45.2 for [enameled cast-iron] [enameled-steel] bathtubs.

            - Standard: ANSI Z124.1.2 for [FRP] [PMMA] plastic bathtubs.

            - Bathing Surface: Slip resistant in accordance with ASTM F 462.

            - Size: [48 by 30 inches (1219 by 762 mm)] [60 by 30 inches (1525 by 762 mm)] [66 by 30 inches (1680 by 762 mm)] <Insert dimensions> [with front apron] [drop-in type].

            - Color: [White] <Insert color>.

            - Drain Location: [Left] [Right] end.

            - Drain: NPS 1-1/2 (DN 40); chrome-plated brass, pop-up waste, and overflow.

          - Faucet: <Insert faucet designation from "Bathtub Faucets" Article>.

          - Supply Fittings: Included in faucet.

          - Tub Filler: Chrome-plated-brass diverter spout.

          - Waste Fittings:

            - Standard: ASME A112.18.2/CSA B45.125.2.

            - Drain: Stainless steel or chrome-plated brass, removable strainer.

            - Overflow: Chrome-plated-brass escutcheon with toggle drain-plug device.

Retain one of two "Drain Piping" subparagraphs below.

            - Drain Piping: NPS 1-1/2 (DN 40) cast-brass overflow, P-trap, and waste.

            - Drain Piping: Schedule 40 [ABS] [or] [PVC], NPS 1-1/2 (DN 40) P-trap, and waste.

      - BATHTUB FAUCETS

        - NSF Standard: Comply with NSF/ANSI 61 Annex G, "Drinking Water System Components - Health Effects," for faucet materials that will be in contact with potable water.

Copy "Bathtub Faucets" Paragraph below and re-edit for each product.

Insert drawing designation. Use these designations on Drawings to identify each bathtub faucet.

        - Bathtub Faucets <Insert drawing designation>: Single handle, [pressure balance] [thermostatic].

          - Pressure-Balance Faucets:

Retain "Manufacturers" Subparagraph and list of manufacturers below to require products from manufacturers listed or a comparable product from other manufacturers.

            - Manufacturers: Subject to compliance with requirements, provide products by one of the following:

              - American Standard.

              - Briggs Plumbing Products, Inc.

              - Central Brass Company.

              - Danze, Inc.

              - FNW; Ferguson Enterprises, Inc.

              - Gerber Plumbing Fixtures LLC.

              - Kohler Co.

              - Lawler Manufacturing Company, Inc.

              - Leonard Valve Company.

              - Matco-Norca.

              - Moen Incorporated.

              - POWERS; a WATTS brand.

              - Speakman Company.

              - St. Thomas Creations.

              - Sterling.

              - Symmons Industries, Inc.

              - T&S Brass and Bronze Works, Inc.

              - Wolverine Brass, Inc.

              - Zurn Industries, LLC.

              - <Insert manufacturer's name>.

          - Thermostatic Faucets:

Retain "Manufacturers" Subparagraph and list of manufacturers below to require products from manufacturers listed or a comparable product from other manufacturers.

            - Manufacturers: Subject to compliance with requirements, provide products by one of the following:

              - American Standard.

              - Lawler Manufacturing Company, Inc.

              - Leonard Valve Company.

              - POWERS; a WATTS brand.

              - <Insert manufacturer's name>.

          - Standards: ASME A112.18.1/CSA B125.1 and ASSE 1016.

          - Faucet:

            - Body Material: Solid brass.

            - Finish: Polished chrome plate.

            - Maximum Flow Rate: [2.5 gpm (9.5 L/min.)] <Insert value> unless otherwise indicated.

            - Mounting: [Concealed] [Exposed].

            - Operation: Single handle, [push-pull] [or] [twist or rotate] control, with hot- and cold-water indicators.

            - Antiscald Device: [Integral with mixing valve] [Separate unit] [Not required].

            - Check Stops: Check-valve type, integral with or attached to body; on hot- and cold-water supply connections.

            - Diverter: In-tub filler spout.

            - Supply Connections: NPS 1/2 (DN 15).

          - Shower Head:

            - Standard: ASME A112.18.1/CSA B125.1.

            - Type: [Ball joint with arm and flange] [Without ball joint, but with arm and flange] [Ball joint and head integral with mounting flange] [Integral with mounting flange] [Hand shower. Include wall-mounting device] <Insert type>.

Retain "Backflow-Prevention Device" Subparagraph below with hand-shower type.

            - Backflow-Prevention Device: ASSE 1014.

            - Shower Head Material: Metallic with chrome-plated finish.

            - Spray Pattern: [Adjustable] [Fixed].

            - Integral Volume Control: [Not required] [Required].

Retain "Shower-Arm, Flow-Control Fitting" Subparagraph below only if retaining option that includes arm and flange in "Type" Subparagraph.

            - Shower-Arm, Flow-Control Fitting: [Not required] [1.5 gpm (5.7 L/min.)] [2.0 gpm (7.6 L/min.)].

Verify availability of feature in "Temperature Indicator" Subparagraph below if required.

            - Temperature Indicator: [Integral with faucet] [Not required].

          - Bathtub Filler Spout: Chrome-plated brass.

      - WHIRLPOOL BATHS

Whirlpool baths in this article are usually the drop-in-type mounted in a base unit. Some units are freestanding with a front apron. Both types require access to equipment.

Copy "Whirlpool Baths" Paragraph below and re-edit for each product.

Insert drawing designation. Use these designations on Drawings to identify each hydromassage bathtub.

        - Whirlpool Baths <Insert drawing designation>: [Enameled cast-iron] [Enameled-steel] [FRP] [PMMA] hydromassage bathtub.

          - Enameled Cast-Iron Whirlpool Baths:

Retain "Manufacturers" Subparagraph and list of manufacturers below to require products from manufacturers listed or a comparable product from other manufacturers.

            - Manufacturers: Subject to compliance with requirements, provide products by the following:

              - Kohler Co.

              - <Insert manufacturer's name>.

          - Enameled-Steel Whirlpool Baths:

Retain "Manufacturers" Subparagraph and list of manufacturers below to require products from manufacturers listed or a comparable product from other manufacturers.

            - Manufacturers: Subject to compliance with requirements, provide products by one of the following:

              - American Standard.

              - Mansfield Plumbing Products LLC.

              - <Insert manufacturer's name>.

          - FRP Whirlpool Baths:

Retain "Manufacturers" Subparagraph and list of manufacturers below to require products from manufacturers listed or a comparable product from other manufacturers.

            - Manufacturers: Subject to compliance with requirements, provide products by one of the following:

              - Best Bath Systems.

              - Clarion Bathware.

              - MAAX.

              - Praxis Industries, LLC.

              - Sterling.

              - <Insert manufacturer's name>.

          - PMMA (Acrylic) Whirlpool Baths:

Retain "Manufacturers" Subparagraph and list of manufacturers below to require products from manufacturers listed or a comparable product from other manufacturers.

            - Manufacturers: Subject to compliance with requirements, provide products by one of the following:

              - American Standard.

              - American Whirlpool.

              - Aqua Glass Corporation.

              - Aquatic Industries, Inc.

              - BainUltra, Inc.

              - Benjamin Manufacturing Co., Inc.

              - Clarion Bathware.

              - Clarke Products, Inc.

              - Crane Plumbing, L.L.C.

              - Eljer, Inc.

              - Florestone Products Co., Inc.

              - Jacuzzi Inc.

              - Jason International, Inc.

              - Jetta Corporation.

              - Kohler Co.

              - LASCO Bathware.

              - Lyons Industries, Inc.

              - MAAX.

              - Mansfield Plumbing Products LLC.

              - Praxis Industries, LLC.

              - Royal Baths Manufacturing Co.

              - Southland Spa & Sauna Inc.

              - <Insert manufacturer's name>.

Retain one of two "Standards" subparagraphs below.

          - Standards: ASME A112.19.1/CSA B45.2 and UL 1795 for enameled [-steel] [cast-iron] hydromassage bathtubs.

          - Standards: ANSI Z124.1.2 and UL 1795 for [FRP] [PMMA] hydromassage bathtubs.

          - General: Bathtub with air-entrained, water-jet nozzles and water circulation.

          - Fixture:

            - Seating Capacity: [One] [Two] <Insert number> person(s).

            - Bathing Surface: Slip resistant in accordance with ASTM F 462.

            - Size: [60 by 30 inches (1525 by 762 mm)] [66 by 30 inches (1680 by 762 mm)] [60 by 42 inches (1525 by 1065 mm)] <Insert dimensions>.

            - Nominal Water Capacity: <Insert gal. (L)>.

            - Apron: Matching unit, covering exposed front and sides, and with access panel.

            - Color: [White] <Insert color>.

            - Drain Location: [Left] [Right] end.

Insert specific controls required for "Controls" Subparagraph below.

            - Controls: For pump[, timer,] [and water heater].

          - Faucet:

            - NSF Standard: Comply with NSF/ANSI 61 Annex G, "Drinking Water System Components - Health Effects," for faucet materials that will be in contact with potable water.

            - Description: Fixture manufacturer's chrome-plated, solid-brass [individual valves] [mixing valve] with over-rim tub filler.

            - Faucet Standard: ASME A112.18.1/CSA B125.1.

          - Supplies: NPS 1/2 (DN 15) copper tubing with ball, gate, or globe valves.

          - Waste Fittings:

            - Standard: ASME A112.18.2/CSA B45.125.2.

            - Size: NPS 1-1/2 (DN 40).

            - Drain: Chrome-plated exposed parts; brass pop-up waste and overflow.

Retain one of two "Drain Piping" subparagraphs below.

            - Drain Piping: Cast- and tubular-brass overflow, P-trap, and waste.

            - Drain Piping: Schedule 40 [ABS] [or] [PVC], overflow, P-trap, and waste.

          - Water-Circulating System: Electric circulating pump complying with ASME A112.19.7 and with UL 1951, and listed and labeled as defined in NFPA 70, by a qualified testing agency, and marked for intended location and application and plastic piping.

"Water Heater" Subparagraph below is a useful, but unnecessary, feature.

          - Water Heater: Electric, inline complying with UL 1951, and listed and labeled as defined in NFPA 70, by a qualified testing agency, and marked for intended location and application.

          - Unit Electrical Characteristics:

            - Volts: [120] [240] [277] [480] <Insert number> V.

            - Phase(s): [Single] [Three].

            - Hertz: [60] <Insert number> Hz.

            - Full-Load Amperes: <Insert value> A.

            - Minimum Circuit Ampacity: <Insert value> A.

            - Maximum Overcurrent Protection: <Insert value> A.

Copy "Whirlpool Baths" Paragraph below and re-edit for each product.

Insert drawing designation. Use these designations on Drawings to identify each air-massage bathtub.

        - Whirlpool Baths <Insert drawing designation>: PMMA air-massage bathtub.

Retain "Manufacturers" Subparagraph and list of manufacturers below to require products from manufacturers listed or a comparable product from other manufacturers.

          - Manufacturers: Subject to compliance with requirements, provide products by one of the following:

            - Acryline USA, Inc.

            - American Standard.

            - American Whirlpool.

            - Aqua Glass Corporation.

            - Aquatic Industries, Inc.

            - BainUltra, Inc.

            - Jacuzzi Inc.

            - Jason International, Inc.

            - Jetta Corporation.

            - <Insert manufacturer's name>.

          - Standards: ANSI Z124.1.2 and UL 1795.

          - General: Air-massage bathtub with air-injection nozzles.

          - Fixture:

            - Seating Capacity: [One] [Two] <Insert number> person(s).

            - Bathing Surface: Slip resistant in accordance with ASTM F 462.

            - Size: [60 by 30 inches (1525 by 762 mm)] [66 by 30 inches (1680 by 762 mm)] [60 by 42 inches (1525 by 1065 mm)] <Insert dimensions>.

            - Nominal Water Capacity: <Insert gal. (L)>.

            - Apron: Matching unit, covering exposed front and sides, and with access panel.

            - Color: [White] <Insert color>.

            - Drain Location: [Left] [Right] end.

Insert specific controls required for "Controls" Subparagraph below.

          - Controls: For blower[, timer,] [and water heater].

          - Faucet:

            - NSF Standard: Comply with NSF/ANSI 61 Annex G, "Drinking Water System Components - Health Effects," for faucet materials that will be in contact with potable water.

            - Description: Fixture manufacturer's chrome-plated, solid-brass [individual valves] [mixing valve] with over-rim tub filler.

            - Faucet Standard: ASME A112.18.1/CSA B125.1.

          - Supplies: NPS 1/2 (DN 15) copper tubing with ball, gate, or globe valves.

          - Waste Fittings:

            - Standard: ASME A112.18.2/CSA B45.125.2.

            - Size: NPS 1-1/2 (DN 40).

            - Drain: Chrome-plated exposed parts; brass pop-up waste and overflow.

Retain one of two "Drain Piping" subparagraphs below.

            - Drain Piping: Cast- and tubular-brass overflow, P-trap, and waste.

            - Drain Piping: Schedule 40 [ABS] [or] [PVC], overflow, P-trap, and waste.

          - Water-Circulating System: Electric circulating pump, complying with ASME A112.19.7 and with UL 1951, and listed and labeled as defined in NFPA 70, by a qualified testing agency, and marked for intended location and application and plastic piping.

"Water Heater" Subparagraph below is a useful, but unnecessary, feature.

          - Water Heater: Electric, inline complying with UL 1951, and listed and labeled as defined in NFPA 70, by a qualified testing agency, and marked for intended location and application.

          - Unit Electrical Characteristics:

            - Volts: [120] [240] [277] [480] <Insert number> V.

            - Phase(s): [Single] [Three].

            - Hertz: [60] <Insert number> Hz.

            - Full-Load Amperes: <Insert value> A.

            - Minimum Circuit Ampacity: <Insert value> A.

            - Maximum Overcurrent Protection: <Insert value> A.

          - Air-Injection System: Electric, [blower] [combination blower/heater] complying with ASME A112.19.7 and with UL 1951, and listed and labeled as defined in NFPA 70, by a qualified testing agency, and marked for intended location and application and plastic piping.

          - Unit Electrical Characteristics:

            - Volts: [120] [240] [277] [480] <Insert value> V.

            - Phase(s): [Single] [Three].

            - Hertz: [60] <Insert value> Hz.

            - Full-Load Amperes: <Insert value> A.

            - Minimum Circuit Ampacity: <Insert value> A.

            - Maximum Overcurrent Protection: <Insert value> A.

      - WALK-IN BATHS

Copy "Walk-in Baths" Paragraph below and re-edit for each product.

Insert drawing designation. Use these designations on Drawings to identify each walk-in bath.

        - Walk-in Baths <Insert drawing designation>: Standard, plastic, with side-entry door, seat, and faucet.

Retain "Manufacturers" Subparagraph and list of manufacturers below to require products from manufacturers listed or a comparable product from other manufacturers.

          - Manufacturers: Subject to compliance with requirements, provide products by one of the following:

            - American Standard.

            - BathEase Inc.

            - Best Bath Systems.

            - BOCA Walk In Tubs.

            - Glamour Baths.

            - Safety Tubs LLC.

            - Serenity Walk In Bathtubs.

            - Superior Living Products.

            - Walk In Bathtub Company (The).

            - <Insert manufacturer's name>.

          - Fixture:

            - Standards: ANSI Z124.1.2.

            - Material: [Plastic] [PMMA] <Insert material>.

            - Nominal Size: Approximately 60 by 32 by 21 inches (1525 by 815 by 535 mm).

            - Skirt: [Front only] [On sides not against walls].

            - Seat: Integral.

            - Grab Bar: Manufacturer's standard.

            - Door: Side opening with rubber sealing gasket.

            - Door Location: [Left] [Right] side.

            - Drain Location: [Left] [Right] end.

          - Faucet:

            - NSF Standard: Comply with NSF/ANSI 61 Annex G, "Drinking Water System Components - Health Effects," for faucet materials that will be in contact with potable water.

            - Description: Fixture manufacturer's chrome-plated, solid-brass [individual valves] [mixing valve] with over-rim tub filler.[ Include diverter and hand shower.]

            - Faucet Standard: ASME A112.18.1/CSA B125.1.

            - Mounting: Deck.

          - Supplies: NPS 1/2 (DN 15) copper tubing with ball, gate, or globe valves.

          - Waste Fittings:

            - Standard: ASME A112.18.2/CSA B45.125.2.

            - Size: NPS 1-1/2 (DN 40).

            - Drain: Chrome-plated exposed parts; brass pop-up waste and overflow.

Retain one of two "Drain Piping" subparagraphs below.

            - Drain Piping: Cast- and tubular-brass overflow, P-trap, and waste.

            - Drain Piping: Schedule 40 [ABS] [or] [PVC], overflow, P-trap, and waste.

          - Water-Circulating System: Electric circulating pump complying with ASME A112.19.7 and with UL 1951, and listed and labeled as defined in NFPA 70, by a qualified testing agency, and marked for intended location and application and plastic piping.

          - Unit Electrical Characteristics:

            - Volts: [120] [240] [277] [480] <Insert value> V.

            - Phase(s): [Single] [Three].

            - Hertz: [60] <Insert value> Hz.

            - Full-Load Amperes: <Insert value> A.

            - Minimum Circuit Ampacity: <Insert value> A.

            - Maximum Overcurrent Protection: <Insert > A.

Copy "Walk-in Baths" Paragraph below and re-edit for each product.

Insert drawing designation. Use these designations on Drawings to identify each walk-in bath.

        - Walk-in Baths <Insert drawing designation>: Plastic, hydromassage, with side-entry door, seat, and faucet.

Retain "Manufacturers" Subparagraph and list of manufacturers below to require products from manufacturers listed or a comparable product from other manufacturers.

          - Manufacturers: Subject to compliance with requirements, provide products by one of the following:

            - American Standard.

            - BathEase Inc.

            - Glamour Baths.

            - Safety Tubs LLC.

            - Walk In Bathtub Company (The).

            - <Insert manufacturer's name>.

          - Fixture:

            - Standards: ANSI Z124.1.2 and UL 1795.

            - Nominal Size: Approximately 60 by 32 by 21 inches (1525 by 815 by 535 mm).

            - Skirt: [Front only] [On sides not against walls].

            - Seat: Integral.

            - Grab Bar: Manufacturer's standard.

            - Door: Side opening with rubber sealing gasket.

            - Door Location: [Left] [Right] side.

            - Drain Location: [Left] [Right] end.

          - Faucet:

            - NSF Standard: Comply with NSF/ANSI 61 Annex G, "Drinking Water System Components - Health Effects," for faucet materials that will be in contact with potable water.

            - Description: Fixture manufacturer's chrome-plated, solid-brass [individual valves] [mixing valve] with over-rim tub filler.[ Include diverter and hand shower.]

            - Faucet Standard: ASME A112.18.1/CSA B125.1.

            - Mounting: Deck.

          - Supplies: NPS 1/2 (DN 15) copper tubing with ball, gate, or globe valves.

          - Waste Fittings:

            - Standard: ASME A112.18.2/CSA B45.125.2.

            - Size: NPS 1-1/2 (DN 40).

            - Drain: Chrome-plated exposed parts; brass pop-up waste and overflow.

Retain one of two "Drain Piping" subparagraphs below.

            - Drain Piping: Cast- and tubular-brass overflow, P-trap, and waste.

            - Drain Piping: Schedule 40 [ABS] [or] [PVC], overflow, P-trap, and waste.

          - Water-Circulating System: Electric circulating pump complying with ASME A112.19.7 and with UL 1951, and listed and labeled as defined in NFPA 70, by a qualified testing agency, and marked for intended location and application and plastic piping.

          - Unit Electrical Characteristics:

            - Volts: [120] [240] [277] [480] <Insert value> V.

            - Phase(s): [Single] [Three].

            - Hertz: [60] <Insert value> Hz.

            - Full-Load Amperes: <Insert value> A.

            - Minimum Circuit Ampacity: <Insert value> A.

            - Maximum Overcurrent Protection: <Insert value> A.

      - BIDETS

Copy "Bidets" Paragraph below and re-edit for each product.

Insert drawing designation. Use these designations on Drawings to identify each bidet.

        - Bidets <Insert drawing designation>:

Retain "Manufacturers" Subparagraph and list of manufacturers below to require products from manufacturers listed or a comparable product from other manufacturers.

          - Manufacturers: Subject to compliance with requirements, provide products by one of the following:

            - American Standard.

            - Briggs Plumbing Products, Inc.

            - Capizzi.

            - Crane Plumbing, L.L.C.

            - Danze, Inc.

            - Duravit USA, Inc.

            - Eljer, Inc.

            - FNW; Ferguson Enterprises, Inc.

            - Gerber Plumbing Fixtures LLC.

            - Kohler Co.

            - Mansfield Plumbing Products LLC.

            - Peerless Pottery Sales, Inc.

            - St. Thomas Creations.

            - TOTO USA, INC.

            - <Insert manufacturer's name>.

          - Fixture:

            - Standard: ASME A112.19.2/CSA B45.1 for vitreous-china bidets.

            - Type: With [spray] [flushing rim] [spray and flushing rim] [no spray].

            - Hole Punching: Number and spacing to match faucet.

            - Mounting: Floor.

            - Overflow: [Not required] [Required].

            - Color: [White] <Insert color>.

          - Faucet:

            - Standard: ASME A112.18.3.

            - Type: Manufacturer's standard with [vacuum breaker, diverter, and submerged spray] [over-rim filling], pop-up drain,[ NPS 1-1/4 (DN 32)] tailpiece, and chrome-plated finish. Include [manufacturer's standard] [cross] [lever] [wrist-blade] <Insert type> handles.

          - Supply Fittings: Comply with requirements in "Supply Fittings" Article.

          - Waste Fittings: Comply with requirements in "Waste Fittings" Article, except faucet includes pop-up drain.

      - LAVATORIES

Copy "Lavatories" Paragraph below and re-edit for each product.

Insert drawing designation. Use these designations on Drawings to identify each lavatory.

        - Lavatories <Insert drawing designation>: [Oval] [Rectangular] [Round], [enameled cast iron] [enameled steel] [PMMA] [solid surface] [stainless steel] [vitreous china], counter mounted.

          - Enameled Cast-Iron Lavatories:

Retain "Manufacturers" Subparagraph and list of manufacturers below to require products from manufacturers listed or a comparable product from other manufacturers.

            - Manufacturers: Subject to compliance with requirements, provide products by one of the following:

              - Commercial Enameling Company.

              - Eljer, Inc.

              - Kohler Co.

              - <Insert manufacturer's name>.

          - Enameled-Steel Lavatories:

Retain "Manufacturers" Subparagraph and list of manufacturers below to require products from manufacturers listed or a comparable product from other manufacturers.

            - Manufacturers: Subject to compliance with requirements, provide products by one of the following:

              - American Standard.

              - Bootz Industries.

              - Briggs Plumbing Products, Inc.

              - Crane Plumbing, L.L.C.

              - Eljer, Inc.

              - Mansfield Plumbing Products LLC.

              - <Insert manufacturer's name>.

          - PMMA (Acrylic) Lavatories:

Retain "Manufacturers" Subparagraph and list of manufacturers below to require products from manufacturers listed or a comparable product from other manufacturers.

            - Manufacturers: Subject to compliance with requirements, provide products by one of the following:

              - Benjamin Manufacturing Co., Inc.

              - Meganite Inc.

              - Royal Baths Manufacturing Co.

              - <Insert manufacturer's name>.

          - Solid-Surface Lavatories:

Retain "Manufacturers" Subparagraph and list of manufacturers below to require products from manufacturers listed or a comparable product from other manufacturers.

            - Manufacturers: Subject to compliance with requirements, provide products by one of the following:

              - American Standard.

              - Bradley Corporation.

              - DuPont.

              - Formica Corporation.

              - Franke Consumer Products, Inc.

              - Inpro Corporation.

              - Meganite Inc.

              - Rynone Manufacturing Corp.

              - Sloan Valve Company.

              - Swan Corporation (The).

              - <Insert manufacturer's name>.

          - Stainless-Steel Lavatories:

Retain "Manufacturers" Subparagraph and list of manufacturers below to require products from manufacturers listed or a comparable product from other manufacturers.

            - Manufacturers: Subject to compliance with requirements, provide products by one of the following:

              - Acorn Engineering Company.

              - Elkay Manufacturing Co.

              - Intersan Manufacturing Company.

              - Just Manufacturing.

              - <Insert manufacturer's name>.

          - Vitreous-China Lavatories:

Retain "Manufacturers" Subparagraph and list of manufacturers below to require products from manufacturers listed or a comparable product from other manufacturers.

            - Manufacturers: Subject to compliance with requirements, provide products by one of the following:

              - American Standard.

              - Barclay Products Limited.

              - Briggs Plumbing Products, Inc.

              - Crane Plumbing, L.L.C.

              - Duravit USA, Inc.

              - Eljer, Inc.

              - Gerber Plumbing Fixtures LLC.

              - Kohler Co.

              - Mansfield Plumbing Products LLC.

              - Peerless Pottery Sales, Inc.

              - Sloan Valve Company.

              - St. Thomas Creations.

              - Sterling.

              - TOTO USA, INC.

              - <Insert manufacturer's name>.

          - Fixture:

Retain one of five "Standard" subparagraphs below.

            - Standard: ASME A112.19.1/CSA B45.2 for enameled [-steel] [cast-iron] lavatories.

            - Standard: ANSI Z124.3 for PMMA lavatories.

            - Standard: ANSI Z124.3 and ANSI/ICPA SS-1 for solid-surface lavatories.

            - Standard: ASME A112.19.3/CSA B45.4 for stainless steel lavatories.

            - Standard: ASME A112.19.2/CSA B45.1 for vitreous-china lavatories.

            - Type: [Flat rim with ledge] [Self-rimming].

Retain "Oval Nominal Size," ''Rectangular Nominal Size," or "Round Nominal Size" Subparagraph below.

            - Oval Nominal Size: [19 by 16 inches (483 by 406 mm)] [20 by 17 inches (508 by 432 mm)] <Insert dimensions>.

            - Rectangular Nominal Size: [18 by 15 inches (457 by 381 mm)] [19 by 16 inches (483 by 406 mm)] [20 by 18 inches (508 by 457 mm)] [24 by 20 inches (610 by 508 mm)] <Insert dimensions>.

            - Round Nominal Size: [18 inches (457 mm)] [19 inches (483 mm)] <Insert dimensions> in diameter.

            - Faucet-Hole Punching: [One hole] [Three holes, 2-inch (51-mm) centers] [Three holes, 4-inch (102-mm) centers].

            - Faucet-Hole Location: [None] [Rim].

            - Color: [Not applicable] [White] <Insert color>.

          - Faucet: <Insert lavatory-faucet designation from "Lavatory Faucets" Article>.

          - Supply Fittings: Comply with requirements in "Supply Fittings" Article.

          - Waste Fittings: Comply with requirements in "Waste Fittings" Article.

Copy "Lavatories" Paragraph below and re-edit for each product.

Insert drawing designation. Use these designations on Drawings to identify each lavatory.

        - Lavatories <Insert drawing designation>: [Oval] [Rectangular], vitreous china, pedestal.

Retain "Manufacturers" Subparagraph and list of manufacturers below to require products from manufacturers listed or a comparable product from other manufacturers.

          - Manufacturers: Subject to compliance with requirements, provide products by one of the following:

            - American Standard.

            - Barclay Products Limited.

            - Briggs Plumbing Products, Inc.

            - Crane Plumbing, L.L.C.

            - Eljer, Inc.

            - Gerber Plumbing Fixtures LLC.

            - Kohler Co.

            - Mansfield Plumbing Products LLC.

            - Peerless Pottery Sales, Inc.

            - St. Thomas Creations.

            - Sterling.

            - TOTO USA, INC.

            - <Insert manufacturer's name>.

          - Fixture:

            - Standard: ASME A112.19.2/CSA B45.1.

Retain "Oval Nominal Size" or "Rectangular Nominal Size" Subparagraph below.

            - Oval Nominal Size: [20 by 17 inches (508 by 432 mm)] [24 by 18 inches (610 by 457 mm)] <Insert dimensions>.

            - Rectangular Nominal Size: [18 by 15 inches (457 by 381 mm)] [19 by 16 inches (483 by 406 mm)] [20 by 18 inches (508 by 457 mm)] [24 by 20 inches (610 by 508 mm)] <Insert dimensions>.

            - Faucet-Hole Punching: [One hole] [Three holes, 2-inch (51-mm) centers] [Three holes, 4-inch (102-mm) centers].

            - Faucet-Hole Location: [Top] [Front wall] [Inclined panel].

            - Color: [White] <Insert color>.

          - Faucet: <Insert sink-faucet designation from "Sink Faucets" Article>.

          - Supply Fittings: Comply with requirements in "Supply Fittings" Article.

          - Waste Fittings: Comply with requirements in "Waste Fittings" Article.

Copy "Lavatories" Paragraph below and re-edit for each product.

Insert drawing designation. Use these designations on Drawings to identify each lavatory.

        - Lavatories <Insert drawing designation>: Oval, [cultured marble,] [solid surface,] integral bowl countertop.

Usually, manufacturers of cultured-marble products only sell locally. See the International Cast Polymer Alliance's website at www.icpa-hq.org or the NAHB Research Center's website at www.nahbrc.org for a list of manufacturers near Project site.

          - Cultured-Marble Countertop Lavatories:

Retain "Manufacturers" Subparagraph and list of manufacturers below to require products from manufacturers listed or a comparable product from other manufacturers.

            - Manufacturers: Subject to compliance with requirements, provide products by one of the following:

              - Capital Marble Creations, Inc.

              - Crane Plumbing, L.L.C.

              - Rynone Manufacturing Corp.

              - Tri-City Marble, Inc.

              - VirginiaMarble Manufacturers, Inc.

              - Waldorf Marble, Inc.

              - <Insert manufacturer's name>.

          - Solid-Surface Countertop Lavatories:

Retain "Manufacturers" Subparagraph and list of manufacturers below to require products from manufacturers listed or a comparable product from other manufacturers.

            - Manufacturers: Subject to compliance with requirements, provide products by one of the following:

              - Aristech Surfaces.

              - DuPont.

              - Formica Corporation.

              - Inpro Corporation.

              - Meganite Inc.

              - Swan Corporation (The).

              - Wilsonart LLC.

              - <Insert manufacturer's name>.

          - Fixture:

            - General: Countertop lavatory with integral bowl(s) for mounting on base unit.

            - Backsplash: [Integral with countertop] [Separate, same material as countertop] [Not required].

            - Overall Rectangular Top Size: [25 by 17 inches (635 by 432 mm)] [31 by 19 inches (787 by 483 mm)] [49 by 22 inches (1245 by 559 mm)] [73 by 22 inches (1854 by 559 mm)] <Insert dimensions> with [one] [two] [three] [four] bowl(s).

            - Bowl Size: Oval [19 by 16 inches (483 by 406 mm)] [20 by 17 inches (508 by 432 mm)] <Insert dimensions>.

            - Faucet-Hole Punching: [One hole] [Three holes, 2-inch (51-mm) centers] [Three holes, 4-inch (102-mm) centers].

            - Faucet-Hole Location: [Countertop] <Insert location>.

            - Color: [White] <Insert color>.

      - LAVATORY FAUCETS

        - NSF Standard: Comply with NSF/ANSI 61 Annex G, "Drinking Water System Components - Health Effects," for faucet materials that will be in contact with potable water.

Copy "Lavatory Faucets" Paragraph below and re-edit for each product.

Insert number drawing designation. Use these designations to identify each lavatory faucet.

        - Lavatory Faucets <Insert drawing designation>: [Single-control mixing] [Single-control nonmixing] [Two-handle mixing] valve.

          - General-Duty, Solid-Brass Faucets:

Retain "Manufacturers" Subparagraph and list of manufacturers below to require products from manufacturers listed or a comparable product from other manufacturers.

            - Manufacturers: Subject to compliance with requirements, provide products by one of the following:

              - American Standard America.

              - Bradley Corporation.

              - BrassTech Inc.

              - Central Brass Company.

              - Chicago Faucets; Geberit Company.

              - Danze, Inc.

              - Delta Faucet Company.

              - Eljer, Inc.

              - Elkay Manufacturing Co.

              - Franke Consumer Products, Inc.

              - Gerber Plumbing Fixtures LLC.

              - Griffin Products, Inc.

              - GROHE America, Inc.

              - Hansgrohe USA.

              - Houzer, Inc.

              - Hydrotek International, Inc.

              - Intersan Manufacturing Company.

              - Just Manufacturing.

              - Kohler Co.

              - Matco-Norca.

              - Moen Incorporated.

              - Price Pfister, Inc.

              - Rohl LLC.

              - Speakman Company.

              - T & S Brass and Bronze Works, Inc.

              - Watermark Designs, Ltd.

              - <Insert manufacturer's name>.

          - General-Duty, Copper- or Brass-Underbody Faucets:

Retain "Manufacturers" Subparagraph and list of manufacturers below to require products from manufacturers listed or a comparable product from other manufacturers.

            - Manufacturers: Subject to compliance with requirements, provide products by one of the following:

              - American Standard America.

              - Briggs Plumbing Products, Inc.

              - Delta Faucet Company.

              - Eljer, Inc.

              - FNW; Ferguson Enterprises, Inc.

              - Gerber Plumbing Fixtures LLC.

              - Moen Incorporated.

              - Sterling.

              - WhiteRock Corp.

              - Wolverine Brass, Inc.

              - <Insert manufacturer's name>.

          - Standard: ASME A112.18.1/CSA B125.1.

          - General: Include hot- and cold-water indicators; coordinate faucet inlets with supplies and fixture holes; coordinate outlet with spout and fixture receptor.

          - Body Material: General-duty, [solid brass] [copper or brass underbody with brass cover plate].

          - Finish: Polished  [chrome plate] [brass].

          - Maximum Flow Rate: [2.2 gpm (8.3 L/min.)] [2.5 gpm (9.5 L/min.)] <Insert value>.

          - Centers: [3-3/8 inches (86 mm)] [4 inches (102 mm)] [6 inches (152 mm)] [8 inches (203 mm)] [Single hole] [Adjustable].

          - Mounting: [Deck, exposed] [Deck, concealed] [Back/wall, exposed] [Back/wall, concealed].

          - Valve Handle(s): [Lever] [Knob] [Knob, nonmetallic] [Cross, four arms] [Wrist blade, 4 inches (102 mm)] [Elbow, 6 inches (152 mm)] [Push button] [Not applicable].

          - Inlet(s): [NPS 3/8 (DN 10) tubing, plain end] [NPS 3/8 (DN 10) tubing, with NPS 1/2 (DN 15) male adaptor] [NPS 1/2 (DN 15) male shank] [NPS 1/2 (DN 15) female shank].

          - Spout: [Rigid] [Swing] [Rigid, gooseneck] [Swivel, gooseneck].

If retaining "Laminar flow" option in "Spout Outlet" Subparagraph below, retain "Laminar-Flow, Faucet-Spout Outlets" Article.

          - Spout Outlet: [Aerator] [Spray] [Laminar flow] [Plain end] [Spray, 0.5 gpm (1.5 L/min.)].

          - Operation: [Compression, manual] [Noncompression, manual] [Sensor] [Self-closing, metering].

          - Drain: [Pop up] [Stopper with chain] [Grid].

      - SHOWERS

Copy "Showers" Paragraph below and re-edit for each product.

Insert drawing designation. Use these designations on Drawings to identify each shower.

        - Showers <Insert drawing designation>: Standard [FRP] [PMMA] with base and faucet.

          - FRP Showers:

Retain "Manufacturers" Subparagraph and list of manufacturers below to require products from manufacturers listed or a comparable product from other manufacturers.

            - Manufacturers: Subject to compliance with requirements, provide products by one of the following:

              - Aqua Glass Corporation.

              - Clarion Bathware.

              - Florestone Products Co., Inc.

              - LASCO Bathware.

              - MAAX.

              - Praxis Industries, LLC.

              - Sterling.

              - Swan Corporation (The).

              - <Insert manufacturer's name>.

          - PMMA (Acrylic) Showers:

Retain "Manufacturers" Subparagraph and list of manufacturers below to require products from manufacturers listed or a comparable product from other manufacturers.

            - Manufacturers: Subject to compliance with requirements, provide products by one of the following:

              - Acryline USA, Inc.

              - Aqua Bath Company, Inc.

              - Aqua Glass Corporation.

              - Aquatic Industries, Inc.

              - Clarion Bathware.

              - Crane Plumbing, L.L.C.

              - Jacuzzi Inc.

              - Kohler Co.

              - LASCO Bathware.

              - MAAX.

              - Praxis Industries, LLC.

              - <Insert manufacturer's name>.

          - Standard: ANSI Z124.1.2.

          - Nominal Size: [36 by 34 inches (915 by 865 mm)] [42 by 36 inches (1065 by 915 mm)] [43 by 39 inches (1090 by 990 mm)] [48 by 34 inches (1219 by 865 mm)] <Insert dimensions>.

Retain first "Surround" Subparagraph below for FRP surround.

          - Surround: One piece[ or sealed, multiple piece].

Retain "Surround" Subparagraph below for PMMA (acrylic) surround.

          - Surround: One piece.

          - Bathing Surface: Slip resistant in accordance with ASTM F 462.

          - Color: [White] <Insert color>.

          - Drain Location: [Left side] [Center] [Right side].

          - Faucet: <Insert faucet designation from "Shower Faucets" Article>.

Copy "Showers" Paragraph below and re-edit for each product.

Insert drawing designation. Use these designations on Drawings to identify each shower.

        - Showers <Insert drawing designation>: Accessible [FRP] [PMMA] with seat, grab bar, base, and faucet.

          - FRP Showers:

Retain "Manufacturers" Subparagraph and list of manufacturers below to require products from manufacturers listed or a comparable product from other manufacturers.

            - Manufacturers: Subject to compliance with requirements, provide products by one of the following:

              - Aqua Glass Corporation.

              - Clarion Bathware.

              - Florestone Products Co., Inc.

              - <Insert manufacturer's name>.

          - PMMA Showers:

Retain "Manufacturers" Subparagraph and list of manufacturers below to require products from manufacturers listed or a comparable product from other manufacturers.

            - Manufacturers: Subject to compliance with requirements, provide products by one of the following:

              - Aqua Bath Company, Inc.

              - Aqua Glass Corporation.

              - Clarion Bathware.

              - Crane Plumbing, L.L.C.

              - <Insert manufacturer's name>.

          - Standards: ANSI Z124.1.2 and ICC/ANSI A117.1 for roll-in shower compartments.

          - Nominal Size: [60 by 36 inches (1525 by 915 mm)] [72 by 36 inches (1830 by 915 mm)] <Insert dimensions>.

Retain first "Surround" Subparagraph below for FRP surround.

          - Surround: One piece[ or sealed, multiple piece].

Retain "Surround" Subparagraph below for PMMA (acrylic) surround.

          - Surround: One piece.

          - Bathing Surface: Slip resistant in accordance with ASTM F 462.

          - Color: [White] <Insert color>.

          - Drain Location: [Left side] [Center] [Right side].

          - Accessibility Options: Grab bar and bench.

          - Faucet: Shower <Insert faucet designation from "Shower Faucets" Article>.

          - Drain: Grid, NPS 2 (DN 50).

Copy "Showers" Paragraph below and re-edit for each product.

Insert drawing designation. Use these designations on Drawings to identify each shower.

        - Showers <Insert drawing designation>: Factory-fabricated cabinet.

Retain "Manufacturers" Subparagraph and list of manufacturers below to require products from manufacturers listed or a comparable product from other manufacturers.

          - Manufacturers: Subject to compliance with requirements, provide products by one of the following:

            - Crane Plumbing, L.L.C.

            - E. L. Mustee & Sons, Inc.

            - Stern-Williams Co., Inc.

            - Swan Corporation (The).

            - <Insert manufacturer's name>.

          - Size: [30 by 30 inches (762 by 762 mm)] [32 by 32 inches (815 by 815 mm)] [36 by 36 inches (915 by 915 mm)] [36 by 39 inches (915 by 990 mm)] [45 by 39 inches (1145 by 990 mm)] <Insert dimensions>.

          - Material: [Steel] [Composite] [Plastic], [front] [corner] [front and rear] access.

          - Color: [Not applicable] <Insert color>.

          - Accessibility Options: Grab bar and bench.

          - Faucet: <Insert faucet designation from "Shower Faucets" Article>.

          - Supplies: NPS 1/2 (DN 15) copper tubing with shutoff valves.

          - Drain: Grid, NPS 2 (DN 50).

      - SHOWER FAUCETS

        - NSF Standard: Comply with NSF/ANSI 61 Annex G, "Drinking Water System Components - Health Effects," for faucet materials that will be in contact with potable water.

Copy "Shower Faucets" Paragraph below and re-edit for each product.

Insert drawing designation. Use these designations in shower description to identify required shower faucet.

        - Shower Faucets <Insert drawing designation>: [Single handle, pressure balance, mixing valve] [Single handle, thermostatic, mixing valve] [Two handle, non-pressure balance and non-thermostatic, mixing valve].

          - Single-Handle, Pressure-Balance Faucets:

Retain "Manufacturers" Subparagraph and list of manufacturers below to require products from manufacturers listed or a comparable product from other manufacturers.

            - Manufacturers: Subject to compliance with requirements, provide products by one of the following:

              - American Standard.

              - Briggs Plumbing Products, Inc.

              - Central Brass Company.

              - Danze, Inc.

              - FNW; Ferguson Enterprises, Inc.

              - Gerber Plumbing Fixtures LLC.

              - Kohler Co.

              - Lawler Manufacturing Company, Inc.

              - Leonard Valve Company.

              - Matco-Norca.

              - Moen Incorporated.

              - POWERS; a WATTS brand.

              - Speakman Company.

              - St. Thomas Creations.

              - Sterling.

              - Symmons Industries, Inc.

              - T&S Brass and Bronze Works, Inc.

              - Wolverine Brass, Inc.

              - Zurn Industries, LLC.

              - <Insert manufacturer's name>.

          - Single-Handle, Thermostatic Faucets:

Retain "Manufacturers" Subparagraph and list of manufacturers below to require products from manufacturers listed or a comparable product from other manufacturers.

            - Manufacturers: Subject to compliance with requirements, provide products by one of the following:

              - American Standard.

              - Lawler Manufacturing Company, Inc.

              - Leonard Valve Company.

              - POWERS; a WATTS brand.

              - <Insert manufacturer's name>.

          - Two-Handle, Non-Pressure-Balance and Non-Thermostatic Mixing Faucets:

Retain "Manufacturers" Subparagraph and list of manufacturers below to require products from manufacturers listed or a comparable product from other manufacturers.

            - Manufacturers: Subject to compliance with requirements, provide products by one of the following:

              - American Standard.

              - Barclay Products Limited.

              - Briggs Plumbing Products, Inc.

              - Danze, Inc.

              - E. L. Mustee & Sons, Inc.

              - FNW; Ferguson Enterprises, Inc.

              - Gerber Plumbing Fixtures LLC.

              - Kohler Co.

              - Mansfield Plumbing Products LLC.

              - Matco-Norca.

              - Paul Decorative Products.

              - Price Pfister, Inc.

              - St. Thomas Creations.

              - Sterling.

              - T&S Brass and Bronze Works, Inc.

              - Wolverine Brass, Inc.

              - Zurn Industries, LLC.

              - <Insert manufacturer's name>.

          - Fixture:

Retain option in "Standard" Subparagraph below for pressure-balance and thermostatic faucets.

            - Standard: ASME A112.18.1/CSA B125.1[ and ASSE 1016].

            - General: Include hot- and cold-water indicators; check stops; and [fixed shower head, arm, and flange] [hand head complying with ASSE 1014 with arm, flange, hose, and bracket]. Coordinate faucet inlets with supplies.

            - Body Material: [Solid brass] <Insert material>[ with nonmetallic trim].

            - Finish: [Polished chrome plate] [Polished brass] <Insert finish>.

            - Maximum Flow Rate: [2.5 gpm (9.5 L/min.)] <Insert value> unless otherwise indicated.

            - Mounting: [Exposed] [Concealed].

            - Backflow-Prevention Device for Handheld Shower: [Required] [Not required].

            - Operation: [Compression] [Noncompression], manual.

            - Antiscald Device: [Integral with mixing valve] [Separate unit] [Not required].

            - Check Stops: Check-valve type, integral with or attached to body; on hot- and cold-water supply connections.

          - Supply Connections: [NPS 1/2 (DN 15)] [NPS 3/4 (DN 20)].

          - Shower Head:

            - Type: [Ball joint] [Without ball joint] [Ball joint and head integral with mounting flange] [Integral with mounting flange] [Handheld, slide-bar mounted] [Handheld, hook mounted].

            - Shower Head Material: [Metallic] [Nonmetallic] [Combined, metallic and nonmetallic] with chrome-plated finish.

            - Spray Pattern: [Fixed] [Adjustable].

            - Integral Volume Control: [Required] [Not required].

            - Shower-Arm, Flow-Control Fitting: [Not required] [1.5 gpm (5.7 L/min.)] [2.0 gpm (7.6 L/min.)].

      - BAR SINKS

Copy "Bar Sinks" Paragraph below and re-edit for each product.

Insert drawing designation. Use these designations on Drawings to identify each bar sink.

        - Bar Sinks <Insert drawing designation>: Single bowl, counter mounted, [enameled cast iron] [enameled steel] [PMMA] [solid surface] [stainless steel].

          - Enameled Cast-Iron Bar Sinks:

Retain "Manufacturers" Subparagraph and list of manufacturers below to require products from manufacturers listed or a comparable product from other manufacturers.

            - Manufacturers: Subject to compliance with requirements, provide products by one of the following:

              - Commercial Enameling Company.

              - Eljer, Inc.

              - Kohler Co.

              - <Insert manufacturer's name>.

          - Enameled-Steel Bar Sinks:

Retain "Manufacturers" Subparagraph and list of manufacturers below to require products from manufacturers listed or a comparable product from other manufacturers.

            - Manufacturers: Subject to compliance with requirements, provide products by one of the following:

              - American Standard.

              - Bootz Industries.

              - Briggs Plumbing Products, Inc.

              - Eljer, Inc.

              - Mansfield Plumbing Products LLC.

              - <Insert manufacturer's name>.

          - PMMA Bar Sinks:

Retain "Manufacturers" Subparagraph and list of manufacturers below to require products from manufacturers listed or a comparable product from other manufacturers.

            - Manufacturers: Subject to compliance with requirements, provide products by one of the following:

              - Benjamin Manufacturing Co., Inc.

              - Gerber Plumbing Fixtures LLC.

              - Lyons Industries, Inc.

              - Meganite Inc.

              - Royal Baths Manufacturing Co.

              - <Insert manufacturer's name>.

          - Solid-Surface Bar Sinks:

Retain "Manufacturers" Subparagraph and list of manufacturers below to require products from manufacturers listed or a comparable product from other manufacturers.

            - Manufacturers: Subject to compliance with requirements, provide products by one of the following:

              - Aristech Surfaces.

              - DuPont.

              - Formica Corporation.

              - Franke Consumer Products, Inc.

              - Inpro Corporation.

              - Meganite Inc.

              - Rynone Manufacturing Corp.

              - Swan Corporation (The).

              - <Insert manufacturer's name>.

          - Stainless Steel Bar Sinks:

Retain "Manufacturers" Subparagraph and list of manufacturers below to require products from manufacturers listed or a comparable product from other manufacturers.

            - Manufacturers: Subject to compliance with requirements, provide products by one of the following:

              - Barclay Products Limited.

              - Elkay Manufacturing Co.

              - Franke Consumer Products, Inc.

              - Griffin Products, Inc.

              - Houzer, Inc.

              - Just Manufacturing.

              - Kohler Co.

              - Moen Incorporated.

              - Revere Sink.

              - Sterling.

              - WhiteRock Corp.

              - <Insert manufacturer's name>.

          - Fixture:

Retain one of five "Standard" subparagraphs below.

            - Standard: ASME A112.19.1/CSA B45.2 for enameled [-steel] [cast-iron] lavatories.

            - Standard: ANSI Z124.3 for PMMA lavatories.

            - Standard: ANSI Z124.3 and ANSI/ICPA SS-1 for solid-surface lavatories.

            - Standard: ASME A112.19.3/CSA B45.4 for stainless steel lavatories.

            - Standard: ASME A112.19.2/CSA B45.1 for vitreous-china lavatories.

            - Type: [Flat rim with ledge] [Self-rimming].

            - Overall Dimensions: <Insert dimensions>.

            - Bowl Dimensions: <Insert dimensions>.

          - Faucet: <Insert sink-faucet designation from "Sink Faucets" Article>.

          - Supply Fittings: Comply with requirements in "Supply Fittings" Article.

          - Waste Fittings: Comply with requirements in "Waste Fittings" Article.

      - KITCHEN SINKS

Copy "Kitchen Sinks" Paragraph below and re-edit for each product.

Insert drawing designation. Use these designations on Drawings to identify each kitchen sink.

        - Kitchen Sinks <Insert drawing designation>: [One] [Two] [Three] bowl(s), counter mounted, [enameled cast iron] [enameled steel] [PMMA] [solid surface] [stainless steel].

          - Enameled Cast-Iron Kitchen Sinks:

Retain "Manufacturers" Subparagraph and list of manufacturers below to require products from manufacturers listed or a comparable product from other manufacturers.

            - Manufacturers: Subject to compliance with requirements, provide products by one of the following:

              - Commercial Enameling Company.

              - Eljer, Inc.

              - Kohler Co.

              - <Insert manufacturer's name>.

          - Enameled-Steel Kitchen Sinks:

Retain "Manufacturers" Subparagraph and list of manufacturers below to require products from manufacturers listed or a comparable product from other manufacturers.

            - Manufacturers: Subject to compliance with requirements, provide products by one of the following:

              - American Standard.

              - Bootz Industries.

              - Briggs Plumbing Products, Inc.

              - Eljer, Inc.

              - Mansfield Plumbing Products LLC.

              - <Insert manufacturer's name>.

          - PMMA Kitchen Sinks:

Retain "Manufacturers" Subparagraph and list of manufacturers below to require products from manufacturers listed or a comparable product from other manufacturers.

            - Manufacturers: Subject to compliance with requirements, provide products by one of the following:

              - Gerber Plumbing Fixtures LLC.

              - Lyons Industries, Inc.

              - <Insert manufacturer's name>.

          - Solid-Surface Kitchen Sinks:

Retain "Manufacturers" Subparagraph and list of manufacturers below to require products from manufacturers listed or a comparable product from other manufacturers.

            - Manufacturers: Subject to compliance with requirements, provide products by one of the following:

              - Aristech Surfaces.

              - DuPont.

              - Formica Corporation.

              - Franke Consumer Products, Inc.

              - Inpro Corporation.

              - Rynone Manufacturing Corp.

              - Swan Corporation (The).

              - Wilsonart LLC.

              - <Insert manufacturer's name>.

          - Stainless-Steel Kitchen Sinks:

Retain "Manufacturers" Subparagraph and list of manufacturers below to require products from manufacturers listed or a comparable product from other manufacturers.

            - Manufacturers: Subject to compliance with requirements, provide products by one of the following:

              - Elkay Manufacturing Co.

              - Franke Consumer Products, Inc.

              - Griffin Products, Inc.

              - Houzer, Inc.

              - Just Manufacturing.

              - Kohler Co.

              - Revere Sink.

              - Sterling.

              - WhiteRock Corp.

              - <Insert manufacturer's name>.

          - Fixture:

Retain one of four "Standard" subparagraphs below.

            - Standard: ASME A112.19.1/CSA B45.2 for enameled [-steel] [cast-iron] kitchen sinks.

            - Standard: ANSI Z124.3 for PMMA kitchen sinks.

            - Standard: ANSI Z124.3 and ANSI/ICPA SS-1 for solid-surface kitchen sinks.

            - Standard: ASME A112.19.3/CSA B45.4 for stainless steel kitchen sinks.

            - Overall Dimensions: <Insert dimensions>.

Retain "Metal Thickness" Subparagraph below for stainless steel sinks.

            - Metal Thickness: [0.038 inch (1.0 mm)] [0.050 inch (1.3 mm)] <Insert dimension>.

Retain "Bowl" Subparagraph below for single-bowl sinks.

            - Bowl:

              - Dimensions: <Insert dimensions>.

              - Drain: 3-1/2-inch (89-mm) [crumb cup] [grid] [outlet for disposer].

                - Location: [Centered in bowl] [Near back of bowl] <Insert location>.

Retain "Left Bowl," "Right Bowl," and "Center Bowl" subparagraphs as required for multibowl sinks.

            - Left Bowl:

              - Dimensions: <Insert dimensions>.

              - Drain: 3-1/2-inch (89-mm) [crumb cup] [grid] [grid with offset waste] [outlet for disposer].

                - Location: [Centered in bowl] [Near back of bowl] <Insert location>.

            - Right Bowl:

              - Dimensions: <Insert dimensions>.

              - Drain: 3-1/2-inch (89-mm) [crumb cup] [grid] [grid with offset waste] [outlet for disposer].

                - Location: [Centered in bowl] [Near back of bowl] <Insert location>.

            - Center Bowl:

              - Dimensions: <Insert dimensions>.

              - Drain: [1-1/2-inch (38-mm)] [3-1/2-inch (89-mm)] [crumb cup] [grid] [grid with offset waste].

                - Location: [Centered in bowl] <Insert location>.

          - Faucet: <Insert sink-faucet designation from "Sink Faucets" Article>.

          - Supply Fittings: Comply with requirements in "Supply Fittings" Article.

          - Waste Fittings: Comply with requirements in "Waste Fittings" Article, except include continuous waste for multibowl sinks.

            - Disposer: [Not required] <Insert disposer designation from "Disposers" Article>.

            - Dishwasher Air-Gap Fitting: [Not required] [Comply with requirements in "Dishwasher Air-Gap Fittings" Article].

            - Hot-Water Dispenser: [Not required] [Comply with requirements in "Hot-Water Dispensers" Article].

      - LAUNDRY TRAYS

Copy "Laundry Trays" Paragraph below and re-edit for each product.

Insert drawing designation. Use these designations on Drawings to identify each laundry tray.

        - Laundry Trays <Insert drawing designation>: Enameled cast iron, [flat rim ledge] [self-rimming with ledge].

Retain "Manufacturers" Subparagraph and list of manufacturers below to require products from manufacturers listed or a comparable product from other manufacturers.

          - Manufacturers: Subject to compliance with requirements, provide products by one of the following:

            - Commercial Enameling Company.

            - Eljer, Inc.

            - Kohler Co.

            - <Insert manufacturer's name>.

          - Fixture:

            - Standard: ASME A112.19.1/CSA B45.2.

            - Nominal Size: [24 by 21 inches (610 by 535 mm)] [24 by 23 inches (610 by 584 mm)] <Insert dimensions>.

            - Color: [White] <Insert color>.

            - Mounting: [Freestanding on manufacturer's standard metal stand] [Flush, counter mounted with stainless steel ring and sealant or on-counter mounted with sealant].

          - Faucet: <Insert sink-faucet designation from "Sink Faucets" Article>.

          - Supply Fittings: Comply with requirements in "Supply Fittings" Article.

          - Waste Fittings: Comply with requirements in "Waste Fittings" Article.

Copy "Laundry Trays" Paragraph below and re-edit for each product.

Insert drawing designation. Use these designations on Drawings to identify each laundry tray.

        - Laundry Trays <Insert drawing designation>: Plastic laundry tray.

Retain "Manufacturers" Subparagraph and list of manufacturers below to require products from manufacturers listed or a comparable product from other manufacturers.

          - Manufacturers: Subject to compliance with requirements, provide products by one of the following:

            - Crane Plumbing, L.L.C.

            - E. L. Mustee & Sons, Inc.

            - Florestone Products Co., Inc.

            - FNW; Ferguson Enterprises, Inc.

            - Gerber Plumbing Fixtures LLC.

            - Swan Corporation (The).

            - Zurn Industries, LLC.

            - <Insert manufacturer's name>.

          - Fixture:

            - Standard: IAPMO/ANSI Z124.6.

            - Style: Flat-rim ledge.

Retain "Plastic" option in "Material" Subparagraph below or retain one or more of other options if a specific material is required.

            - Material: [Plastic] [cast polymer] [FRP] [PMMA] [or] [PP].

            - Nominal Size: [24 by 21 inches (610 by 535 mm)] [25 by 22 inches (635 by 559 mm)] <Insert dimensions>.

            - Color: [White] <Insert color>.

            - Mounting: Freestanding on manufacturer's standard legs or separate, painted-steel stand.

          - Faucet: <Insert sink-faucet designation from "Sink Faucets" Article>.

          - Supply Fittings: Comply with requirements in "Supply Fittings" Article.

          - Waste Fittings: Comply with requirements in "Waste Fittings" Article.

      - SINK FAUCETS

        - NSF Standard: Comply with NSF/ANSI 61 Annex G, "Drinking Water System Components - Health Effects," for faucet materials that will be in contact with potable water.

Copy "Sink Faucets" Paragraph below and re-edit for each Product.

Insert drawing designation. Use these designations to identify each sink faucet.

        - Sink Faucets <Insert drawing designation>: [Solid brass] [Copper underbody], [kitchen sink] [bar sink] [laundry tray].

          - General-Duty, Solid-Brass Faucets:

Retain "Manufacturers" Subparagraph and list of manufacturers below to require products from manufacturers listed or a comparable product from other manufacturers.

            - Manufacturers: Subject to compliance with requirements, provide products by one of the following:

              - American Standard America.

              - Bradley Corporation.

              - BrassTech Inc.

              - Briggs Plumbing Products, Inc.

              - Central Brass Company.

              - Chicago Faucets; Geberit Company.

              - Danze, Inc.

              - Delta Faucet Company.

              - Eljer, Inc.

              - Elkay Manufacturing Co.

              - Fisher Manufacturing Co.

              - Franke Consumer Products, Inc.

              - Gerber Plumbing Fixtures LLC.

              - GROHE America, Inc.

              - Hansgrohe USA.

              - Hydrotek International, Inc.

              - Intersan Manufacturing Company.

              - Just Manufacturing.

              - Kohler Co.

              - Moen Incorporated.

              - Price Pfister, Inc.

              - Rohl LLC.

              - Speakman Company.

              - T & S Brass and Bronze Works, Inc.

              - Watermark Designs, Ltd.

              - Wolverine Brass, Inc.

              - <Insert manufacturer's name>.

          - General-Duty, Copper-Underbody Faucets:

Retain "Manufacturers" Subparagraph and list of manufacturers below to require products from manufacturers listed or a comparable product from other manufacturers.

            - Manufacturers: Subject to compliance with requirements, provide products by one of the following:

              - American Standard America.

              - Briggs Plumbing Products, Inc.

              - Delta Faucet Company.

              - Eljer, Inc.

              - FNW; Ferguson Enterprises, Inc.

              - Moen Incorporated.

              - Sterling.

              - WhiteRock Corp.

              - Wolverine Brass, Inc.

              - <Insert manufacturer's name>.

          - Standard: ASME A112.18.1/CSA B125.1.

          - General: Include hot- and cold-water indicators; coordinate faucet inlets with supplies and fixture holes; coordinate outlet with spout and fixture receptor.

          - Kitchen Sink Option: Separate hand spray complying with ASSE 1025.

          - Finish: [Polished chrome plate] [Polished brass] [Nonmetallic] [Polished or rough brass] [Rough brass] <Insert finish>.

Retain second option in "Maximum Flow Rate" Subparagraph below for laundry-tray faucets.

          - Maximum Flow Rate: [2.5 gpm (9.5 L/min.)] [4.0 gpm (15 L/min.)] <Insert value> unless otherwise indicated.

          - Mixing Valve: [Single control] [Two-lever handle].

          - Backflow-Prevention Device for Hand Spray: [Required] [Not required].

          - Centers: [4 inches (102 mm)] [6 inches (152 mm)] [8 inches (203 mm)] [Single hole] [Adjustable].

          - Mounting: [Deck] [Back/wall], [exposed] [concealed].

          - Handle(s): [Lever] [Knob] [Knob, nonmetallic] [Cross, four arms] [Wrist blade, 4 inches (102 mm)] [Elbow, 6 inches (152 mm)] [Not applicable].

          - Spout Type: [Rigid, solid brass] [Rigid, solid brass with wall brace] [Swing, round tubular] [Swing, shaped tube] [Swing, solid brass] [Rigid gooseneck] [Swivel gooseneck].

If retaining "Laminar flow" option in "Spout Outlet" Subparagraph below, retain "Laminar-Flow, Faucet-Spout Outlets" Article.

          - Spout Outlet: [Aerator] [Swivel aerator/spray] [Spray] [Laminar flow] [Plain end].

          - Drain: [Stopper with chain] [Grid] [Lift and turn].

      - DISHWASHER AIR-GAP FITTINGS

Copy "Dishwasher Air-Gap Fittings" Paragraph below and re-edit for each product.

Insert drawing designation. Use these designations to identify each dishwasher air-gap fitting.

        - Dishwasher Air-Gap Fittings <Insert drawing designation>:

Retain "Manufacturers" Subparagraph and list of manufacturers below to require products from manufacturers listed or a comparable product from other manufacturers.

          - Manufacturers: Subject to compliance with requirements, provide products by one of the following:

            - B & K Industries, Inc.

            - BrassCraft Manufacturing Co.; a Masco company.

            - BrassTech Inc.

            - Dearborn Brass.

            - Geberit US.

            - Sioux Chief Manufacturing Company, Inc.

            - WATTS.

            - <Insert manufacturer's name>.

          - Standard: ASSE 1021.

          - Description: Device designed to prevent backflow of contaminated liquid into domestic dishwashers.

          - Material: Plastic body[ with chrome-plated-brass cover].

          - Hose Connections: 5/8-inch- (16-mm-) ID inlet and 7/8-inch- (22-mm-) ID outlet.

          - Capacity: At least 5 gpm (0.32 L/s); at inlet pressure of at least 5 psig (35 kPa) and at temperature of at least 140 deg F (60 deg C).

          - Mounting: Deck.

Retain "Hoses" Subparagraph below if hoses are required.

          - Hoses: Rubber and suitable for temperature of at least 140 deg F (60 deg C).

            - Inlet Hose: 5/8 inch (16 mm) ID and [48 inches (1219 mm)] <Insert dimension> long.

            - Outlet Hose: 7/8 inch (22 mm) ID and [48 inches (1219 mm)] <Insert dimension> long.

      - LAMINAR-FLOW, FAUCET-SPOUT OUTLETS

Retain this article if a laminar-flow, faucet-spout outlet is specified for any lavatory or sink faucet.

Spout outlets in this article are non-aerating, produce less splashing than conventional faucet spouts, and help reduce bacteria growth.

        - NSF Standard: Comply with NSF/ANSI 61 Annex G, "Drinking Water System Components - Health Effects," for faucet-spout outlet materials that will be in contact with potable water.

Retain "Manufacturers" Paragraph and list of manufacturers below to require products from manufacturers listed or a comparable product from other manufacturers.

        - Manufacturers: Subject to compliance with requirements, provide products by one of the following:

          - AM Conservation Group, Inc.

          - Chronomite Laboratories, Inc.

          - NEOPERL, Inc.

          - <Insert manufacturer's name>.

        - Description: Chrome-plated-brass, faucet-spout outlet that produces non-aerating, laminar stream. Include external or internal thread that mates with faucet outlet for attachment to faucets where indicated and flow-rate range that includes flow of faucet.

      - DISPOSERS

Copy "Disposers" Paragraph below and re-edit for each product.

Insert drawing designation. Use these designations on Drawings to identify each disposer.

        - Disposers <Insert drawing designation>: [Batch] [Continuous]-feed household, food waste.

          - Batch-Feed Disposers:

Retain "Manufacturers" Subparagraph and list of manufacturers below to require products from manufacturers listed or a comparable product from other manufacturers.

            - Manufacturers: Subject to compliance with requirements, provide products by one of the following:

              - American Standard.

              - Franke Consumer Products, Inc.

              - InSinkErator; a division of Emerson Electric Co.

              - KitchenAid; a division of Whirlpool Corporation.

              - Maytag; a division of Whirlpool Corporation.

              - <Insert manufacturer's name>.

          - Continuous-Feed Disposers:

Retain "Manufacturers" Subparagraph and list of manufacturers below to require products from manufacturers listed or a comparable product from other manufacturers.

            - Manufacturers: Subject to compliance with requirements, provide products by one of the following:

              - American Standard.

              - Anaheim Manufacturing, Inc.

              - Franke Consumer Products, Inc.

              - InSinkErator; a division of Emerson Electric Co.

              - KitchenAid; a division of Whirlpool Corporation.

              - Maytag; a division of Whirlpool Corporation.

              - WhiteRock Corp.

              - <Insert manufacturer's name>.

          - Standards: ASSE 1008 and UL 430, and listed and labeled as defined in NFPA 70, by a qualified testing agency, and marked for intended location and application.

          - General: Include reset button; wall switch; corrosion-resistant chamber with jam-resistant, cutlery- or stainless steel grinder or shredder; NPS 1-1/2 (DN 40) outlet; quick-mounting, stainless steel sink flange; antisplash guard; and combination cover/stopper.

          - Model: [Not applicable] [Sound-insulated chamber] [Sound-insulated chamber and stainless steel outer shell].

          - Motor: 115-V ac, 1725 rpm, [1/3] [1/2] [3/4] [1] hp with overload protection.

      - HOT-WATER DISPENSERS

        - NSF Standard: Comply with NSF/ANSI 61 Annex G, "Drinking Water System Components - Health Effects," for faucet materials that will be in contact with potable water.

Copy "Hot-Water Dispensers" Paragraph below and re-edit for each product.

Insert drawing designation. Use these designations on Drawings to identify each hot-water dispenser.

        - Hot-Water Dispensers <Insert drawing designation>: [Gooseneck spout with lever handle] [Spout with twist knob or push button], flow control.

          - Gooseneck Spout with Lever-Handle, Flow-Control, Hot-Water Dispensers:

Retain "Manufacturers" Subparagraph and list of manufacturers below to require products from manufacturers listed or a comparable product from other manufacturers.

            - Manufacturers: Subject to compliance with requirements, provide products by one of the following:

              - Anaheim Manufacturing, Inc.

              - Elkay Manufacturing Co.

              - Franke Consumer Products, Inc.

              - InSinkErator; a division of Emerson Electric Co.

              - Just Manufacturing.

              - KitchenAid.

              - <Insert manufacturer's name>.

          - Spout with Twist-Knob or Push-Button, Flow-Control, Hot-Water Dispensers:

Retain "Manufacturers" Subparagraph and list of manufacturers below to require products from manufacturers listed or a comparable product from other manufacturers.

            - Manufacturers: Subject to compliance with requirements, provide products by one of the following:

              - Anaheim Manufacturing, Inc.

              - Elkay Manufacturing Co.

              - InSinkErator; a division of Emerson Electric Co.

              - Just Manufacturing.

              - KitchenAid; a division of Whirlpool Corporation.

              - <Insert manufacturer's name>.

          - Standard: ASSE 1023.

          - General: Household-type dispenser with instant on-off control; insulated, corrosion-resistant-metal storage tank that is open to atmosphere; electric heating element; chrome-plated faucet or spout; removable strainer; thermostat control for water temperature up to 190 deg F (88 deg C); and thermal-overload protection.

          - Storage Tank Capacity: 0.5 gal. (1.5 L).

          - Heating Element: 750 W minimum, 115-V ac.

      - WATER CLOSETS

Copy "Water Closets" Paragraph below and re-edit for each product.

Insert drawing designation. Use these designations on Drawings to identify each water closet.

        - Water Closets <Insert drawing designation>: Floor mounted, floor outlet, close coupled (gravity tank), vitreous china.

Retain "Manufacturers" Subparagraph and list of manufacturers below to require products from manufacturers listed or a comparable product from other manufacturers.

          - Manufacturers: Subject to compliance with requirements, provide products by one of the following:

            - American Standard.

            - Barclay Products Limited.

            - Briggs Plumbing Products, Inc.

            - Capizzi.

            - Crane Plumbing, L.L.C.

            - Duravit USA, Inc.

            - Eljer, Inc.

            - FNW; Ferguson Enterprises, Inc.

            - Gerber Plumbing Fixtures LLC.

            - Kohler Co.

            - Lamosa Corp.

            - Mansfield Plumbing Products LLC.

            - Peerless Pottery Sales, Inc.

            - St. Thomas Creations.

            - TOTO USA, INC.

            - Zurn Industries, LLC.

            - <Insert manufacturer's name>.

          - Bowl:

            - Standards: ASME A112.19.2/CSA B45.1, ASME A112.19.5, and ASSE 1037.

            - Bowl Type: [Siphon jet] <Insert type>.

            - Height: [Standard] [Child] [People with disabilities/elderly].

            - Rim Contour: [Elongated] [Regular].

Retain first option in "Water Consumption" Subparagraph below unless special, low-consumption design is required.

            - Water Consumption: [Water saving] [Low].

            - Color: [White] <Insert color>.

          - Toilet Seat: <Insert toilet-seat designation from "Toilet Seats" Article>.

          - Supply Fittings:

            - Standard: ASME A112.18.1/CSA B125.1.

            - Supply Piping: Chrome-plated-brass pipe or chrome-plated-copper tube matching water-supply piping size. Include chrome-plated wall flange.

            - Stop: Chrome-plated-brass, one-quarter-turn, ball-type or compression stop with inlet connection matching water-supply piping type and size.

              - Operation: [Loose key] [Wheel handle] <Insert type>.

            - Riser:

              - Size: [NPS 3/8 (DN 10)] [NPS 1/2 (DN 15)].

              - Material: [Chrome-plated, soft-copper flexible tube] [ASME A112.18.6, braided- or corrugated-stainless steel flexible hose] riser.

          - Closet Flange:

Retain "Basis-of-Design Product" Subparagraph and list of manufacturers below to require a specific product or a comparable product from manufacturers listed.

            - Basis-of-Design Product: Subject to compliance with requirements, provide Oatey; Closet Flange or comparable product by one of the following:

              - <Insert manufacturer's name>.

            - Size: [4 inches 102 mm)] [3 to 4 inches (76 to192 mm)]

            - Material: [ABS] [Cast iron] [PVC].

            - Testing: [Cap] [No cap].

            - Ring: Stainless steel, one-piece, (stainless steel replacement ring is available)

            - Type: [Spigot] [Long pattern] [Level-fit] [Inside fit] [Offset] [Short slot].

Copy "Water Closets" Paragraph below and re-edit for each product.

Insert drawing designation. Use these designations on Drawings to identify each water closet.

        - Water Closets <Insert drawing designation>: Floor mounted, floor outlet, close coupled (flushometer tank), vitreous china.

          - Sloan's "Flushmate" Flushometer-Tank, 1.0-Gal./Flush (3.8-L/Flush) Water Closets:

Retain "Manufacturers" Subparagraph and list of manufacturers below to require products from manufacturers listed or a comparable product from other manufacturers.

            - Manufacturers: Subject to compliance with requirements, provide products by one of the following:

              - Capizzi.

              - Kohler Co.

              - St. Thomas Creations.

              - Zurn Industries, LLC.

              - <Insert manufacturer's name>.

          - Sloan's "Flushmate" Flushometer-Tank, 1.6-Gal./Flush (6-L/Flush) Water Closets:

Retain "Manufacturers" Subparagraph and list of manufacturers below to require products from manufacturers listed or a comparable product from other manufacturers.

            - Manufacturers: Subject to compliance with requirements, provide products by one of the following:

              - American Standard.

              - FNW; Ferguson Enterprises, Inc.

              - Gerber Plumbing Fixtures LLC.

              - Kohler Co.

              - Mansfield Plumbing Products LLC.

              - St. Thomas Creations.

              - Zurn Industries, LLC.

              - <Insert manufacturer's name>.

          - Geberit's "PF/2 Energizer" Flushometer-Tank, 1.6-Gal./Flush (6-L/Flush) Water Closets:

Retain "Manufacturers" Subparagraph and list of manufacturers below to require products from manufacturers listed or a comparable product from other manufacturers.

            - Manufacturers: Subject to compliance with requirements, provide products by one of the following:

              - Crane Plumbing, L.L.C.

              - Eljer, Inc.

              - Peerless Pottery Sales, Inc.

              - <Insert manufacturer's name>.

          - Briggs's Vacuum-Assist, Flushometer-Tank, 1.6-Gal./Flush (6-L/Flush) Water Closets:

Retain "Manufacturers" Subparagraph and list of manufacturers below to require products from manufacturers listed or a comparable product from other manufacturers.

            - Manufacturers: Subject to compliance with requirements, provide products by the following:

              - Briggs Plumbing Products, Inc.

              - <Insert manufacturer's name>.

          - Bowl:

            - Standards: ASME A112.19.2/CSA B45.1 and ASME A112.19.5.

            - Bowl Type: [Siphon jet] <Insert type>.

            - Height: [Standard] [Child] [People with disabilities/elderly].

            - Rim Contour: [Elongated] [Regular].

Retain first option in "Water Consumption" Subparagraph below unless special, low-consumption design is required.

            - Water Consumption: [Water saving] [Low].

            - Color: [White] <Insert color>.

          - Toilet Seat: <Insert toilet-seat designation from "Toilet Seats" Article>.

          - Supply Fittings:

            - Standard: ASME A112.18.1/CSA B125.1.

            - Supply Piping: Chrome-plated-brass pipe or chrome-plated-copper tube matching water-supply piping size. Include chrome-plated wall flange.

            - Stop: Chrome-plated-brass, one-quarter-turn, ball-type or compression stop with inlet connection matching water-supply piping type and size.

              - Operation: [Loose key] [Wheel handle] <Insert type>.

            - Riser:

              - Size: [NPS 3/8 (DN 10)] [NPS 1/2 (DN 15)].

              - Material: [Chrome-plated, soft-copper flexible tube] [ASME A112.18.6, braided- or corrugated-stainless-steel flexible hose] riser.

          - Closet Flange:

Retain "Basis-of-Design Product" Subparagraph and list of manufacturers below to require a specific product or a comparable product from manufacturers listed.

            - Basis-of-Design Product: Subject to compliance with requirements, provide Oatey; Closet Flange or comparable product by one of the following:

              - <Insert manufacturer's name>.

            - Size: [4 inches 102 mm)] [3 to 4 inches (76 to192 mm)]

            - Material: [ABS] [Cast iron] [PVC].

            - Testing: [Cap] [No cap].

            - Ring: Stainless steel, one-piece, (stainless steel replacement ring available)

            - Type: [Spigot] [Long pattern] [Level fit] [Inside fit] [Offset] [Short slot].

      - TOILET SEATS

Copy "Toilet Seats" Paragraph below and re-edit for each product.

Insert drawing designation. Use these designations on Drawings to identify each toilet seat.

        - Toilet Seats <Insert drawing designation>:

Retain "Manufacturers" Subparagraph and list of manufacturers below to require products from manufacturers listed or a comparable product from other manufacturers.

          - Manufacturers: Subject to compliance with requirements, provide products by one of the following:

            - American Standard.

            - Bemis Manufacturing Company.

            - Centoco Manufacturing Corporation.

            - Church Seats; Bemis Manufacturing Company.

            - Eljer, Inc.

            - FNW; Ferguson Enterprises, Inc.

            - Jones Stephens Corp.

            - Kohler Co.

            - Olsonite Seat Co.

            - Pressalit Inc.

            - Sanderson Plumbing Products, Inc.

            - Sperzel of Lexington.

            - Zurn Industries, LLC.

            - <Insert manufacturer's name>.

          - Standard: IAPMO/ANSI Z124.5.

          - Material: Plastic.

          - Type: [Residential] [Commercial (standard)].

          - Shape: [Regular rim (open front)] [Regular rim (closed front)] [Elongated rim (open front)] [Elongated rim (closed front)].

          - Configuration: [Closed] [Open] front [with] [without] cover.

          - Size: [Elongated] [Regular].

          - Hinge Type: [Check] [Self-sustaining] [Self-sustaining, check] [Self-raising].

          - Hinge Material: [Noncorroding metal] [Plastic].

          - Seat Cover: [Required] [Not required].

          - Color: [White] [Black] <Insert color>.

      - SUPPLY FITTINGS

        - NSF Standard: Comply with NSF/ANSI 61 Annex G, "Drinking Water System Components - Health Effects," for faucet materials that will be in contact with potable water.

        - Standard: ASME A112.18.1/CSA B125.1.

        - [Bidet] [Lavatory] [Bar Sink] [Kitchen Sink] [and] [Laundry Tray] Supply Fittings:

          - Supply Piping: Chrome-plated-brass pipe or chrome-plated-copper tube matching water-supply piping size. Include chrome-plated wall flange.

          - Stops: Chrome-plated-brass, one-quarter-turn, ball-type or compression stop with inlet connection matching water-supply piping type and size.

            - Operation: [Loose key] [Wheel handle] <Insert type>.

          - Risers:

            - Size: NPS 3/8 (DN 10) for [bidets] [and] [lavatories].

            - Size: [NPS 3/8 (DN 10)] [NPS 1/2 (DN 15)] for [bar sinks] [kitchen sinks] [and] [laundry trays].

            - Material: [Chrome-plated, soft-copper flexible tube] [ASME A112.18.6, braided- or corrugated-stainless steel flexible hose] riser.

      - WASTE FITTINGS

        - Standard: ASME A112.18.2/CSA B125.2.

        - Drain: Grid type with NPS 1-1/4 (DN 32) offset tailpiece for accessible lavatories.

        - Drain: Pop-up type with NPS 1-1/4 (DN 32) straight tailpiece as part of faucet for standard lavatories.

        - Drain: Grid type with NPS 1-1/2 (DN 40) offset tailpiece for accessible [bar sinks] [and] [kitchen sinks].

        - Drain: Grid type with NPS 1-1/2 (DN 40) straight tailpiece for standard [bar sinks] [and] [kitchen sinks] [and for laundry trays].

        - Trap:

          - Size: [NPS 1-1/4 (DN 32)] [NPS 1-1/2 by NPS 1-1/4 (DN 40 by DN 32)] [NPS 1-1/2 (DN 40)] for [bidets] [and] [lavatories].

          - Size: NPS 1-1/2 (DN 40) for [bar sinks] [kitchen sinks] [and] [laundry trays].

Retain one of three "Material" subparagraphs below.

          - Material: Chrome-plated, [two-piece, cast-brass trap and swivel elbow with 0.032-inch- (0.83-mm-) thick brass tube to wall] [two-piece, cast-brass trap and ground-joint swivel elbow with 0.032-inch- (0.83-mm-) thick brass tube to wall] [one-piece, cast-brass trap with swivel 0.029-inch- (73-mm-) thick, tubular brass wall bend] <Insert trap type>; and chrome-plated-brass or -steel wall flange.

          - Material: Stainless steel, two-piece trap and swivel elbow with 0.012-inch- (0.30-mm-) thick stainless steel tube to wall; and stainless steel wall flange.

          - Material: ASTM F409 [ABS] [or] [PVC] [one] [two]-piece trap and waste to wall and wall flange.

      - GROUT

        - Standard: ASTM C1107/C1107M, Grade B, post-hardening and volume-adjusting, dry, hydraulic-cement grout.

        - Characteristics: Nonshrink; recommended for interior and exterior applications.

        - Design Mix: 5000-psi (34.5-MPa), 28-day compressive strength.

        - Packaging: Premixed and factory packaged.

- EXECUTION

      - EXAMINATION

        - Examine roughing-in of water-supply and sanitary drainage and vent piping systems to verify actual locations of piping connections before plumbing-fixture installation.

        - Examine walls, floors, cabinets, and counters for suitable conditions where fixtures will be installed.

        - Proceed with installation only after unsatisfactory conditions have been corrected.

      - INSTALLATION

        - Install plumbing fixtures level and plumb in accordance with rough-in drawings.

        - Install floor-mounted water closets on closet flange attachments to drainage piping.

First paragraph below applies to counter-mounting fixtures such as lavatories and sinks.

        - Install counter-mounting fixtures in and attached to casework.

        - Install pedestal lavatories on pedestals and secured to wood blocking in wall.

        - Install water-supply piping with stop on each supply to each fixture to be connected to water distribution piping. Attach supplies to supports or substrate within pipe spaces behind fixtures. Install stops in locations where they can be easily reached for operation.

          - Exception: Use ball or gate valves if supply stops are not specified with fixture. Comply with valve requirements specified in Section 220523.12 "Ball Valves for Plumbing Piping" and Section 220523.15 "Gate Valves for Plumbing Piping."

        - Install tanks for accessible, tank-type water closets with lever handle mounted on wide side of compartment.

        - Install toilet seats on water closets.

        - Install faucet flow-control fittings with specified flow rates and patterns in faucet spouts if faucets are not available with required rates and patterns. Include adapters if required.

        - Install shower flow-control fittings with specified maximum flow rates in shower arms.

        - Install traps on fixture outlets.

          - Exception: Omit trap on [fixtures with integral traps] [indirect wastes unless otherwise indicated].

        - Install disposer in outlet of each sink indicated to have a disposer. Install switch where indicated or in wall adjacent to sink if location is not indicated.

        - Install dishwasher air-gap fitting at each sink indicated to have air-gap fitting. Install [in sink deck] [on countertop at sink] <Insert location>. Connect inlet hose to dishwasher and outlet hose to disposer.

        - Install hot-water dispensers in back top surface of sink or in countertop with spout over sink.

        - Set [bathtubs] [baths] [and] [shower receptors] in leveling bed of cement grout.

Indicate on Drawings those lavatories and sinks that are required to be accessible.

        - Install protective shielding pipe covers and enclosures on exposed supplies and waste piping of accessible [lavatories] [and] [sinks]. Comply with requirements in Section 220719 "Plumbing Piping Insulation."

        - Install wall flanges or escutcheons at piping wall penetrations in exposed, finished locations. Use deep-pattern escutcheons if required to conceal protruding fittings. Comply with escutcheon requirements specified in Section 220518 "Escutcheons for Plumbing Piping."

        - Seal joints between plumbing fixtures, counters, floors, and walls using sanitary-type, one-part, mildew-resistant silicone sealant. Match sealant color to fixture color. Comply with sealant requirements specified in Section 079200 "Joint Sealants."

      - CONNECTIONS

Coordinate piping installations and specialty arrangements with Drawings and with requirements specified in piping systems. If Drawings are explicit enough, these requirements may be reduced or omitted.

        - Connect fixtures with water supplies, stops, and risers, and with traps, soil, waste, and vent piping. Use size fittings required to match fixtures.

        - Comply with water piping requirements specified in Section 221116 "Domestic Water Piping."

        - Comply with soil and waste piping requirements specified in Section 221316 "Sanitary Waste and Vent Piping."

        - Install protective shielding pipe covers and enclosures on exposed supplies and waste piping of accessible [lavatories] [and] [sinks]. Comply with requirements in Section 220719 "Plumbing Piping Insulation."

      - ADJUSTING

        - Operate and adjust plumbing fixtures and controls. Replace damaged and malfunctioning fixtures, fittings, and controls.

        - Adjust water pressure at faucets to produce proper flow.

      - CLEANING AND PROTECTION

        - After completing installation of plumbing fixtures, inspect and repair damaged finishes.

        - Clean plumbing fixtures, faucets, and other fittings with manufacturers' recommended cleaning methods and materials.

        - Provide protective covering for installed plumbing fixtures and fittings.

        - Do not allow use of plumbing fixtures for temporary facilities unless approved in writing by Owner.

END OF SECTION 224100